GALAXY AUDIO HDR-2

Software Update

v. OS 03 06

How to install the Software Update:

NOTICE:

1. The software update applies to only the HDR-2.

2. Make sure all the files stored in the built-in Flash memory have been copied to another device before installing the new software update. Remove any SD card from the HDR-2 before the software update is started.

Follow the procedures below to upgrade the software:

- Copy the file "UPGRADE.SMI" to your computer.

- Turn the HDR-2 off (and remove the batteries). Connect it to your computer using the included USB cable

- Copy the file "UPGRADE.SMI" to the USB disk root directory. This will be listed as the first (Flash) “Removable Disk” that appears when the HDR-2 is connected.

- Disconnect the HDR-2 from your computer.

- Connect the HDR-2 to your computer again. The new software will now be upgraded automatically.

- Disconnect the HDR-2 from your computer again.

- Turn on the HDR-2 and check if the new software version has been installed successfully by pressing the MODE- UP - PLAY -and DOWN KEYs sequentially while in idle mode. The software version will be displayed at the bottom right corner as "OS 03 06". This confirms you have upgraded the software successfully.

- To ensure proper operation of the built-in flash memory, please connect the HDR2 with the computer again, and format it as FAT 32.